SWAS ROS 2 Architecture Specification

swas_system Package — BXU + BRU Integration

Project: Automated EV Battery Swapping System (SWAS)
Platform: 2× Raspberry Pi 5 (one on BXU, one on BRU)
ROS 2 Distribution: Humble (recommended, LTS) or Jazzy
Document status: Design specification — pre-implementation

1. Overview and Design Goals

This document defines the ROS 2 architecture for the SWAS battery swapping system. The goal is to wrap the existing, working Python demo into a professional, modular ROS 2 project that:

- Runs the complete swap demo via a single launch file

- Separates concerns clearly (vision, motion, control, orchestration, hardware interfaces)

- Keeps the system easy to extend (new sensors, ML-based perception, simulation)

- Follows industry-standard ROS 2 patterns without over-engineering

The architecture is built around a single ROS 2 package called swas_system, which contains all nodes for both BXU and BRU. Each Pi clones the same repository but launches only its own nodes. Communication between the two Pis happens over the shared local network via standard ROS 2 DDS transport — no Flask or HTTP needed in the final system.

2. System Hardware Summary

BXU (Battery Exchange Unit — the mobile robot)

- Raspberry Pi 5 (main computer)

- Pico #1 (motion): 4× mecanum wheel stepper motors, commanded via USB serial (V vx vy w\n protocol)

- Pico #2 (lifting): controls the battery plate lifting/lowering mechanism via relay board

- Camera #0 and Camera #1 (upward-facing, CSI): used for precise docking alignment under the BRU

- Camera (USB, front-facing): used for front approach and alignment to the BRU

- ArUco markers on BRU used as visual reference (to be replaced with ML perception later)

BRU (Battery Receiving Unit — the fixed station)

- Raspberry Pi 5

- Lock motor (controlled via relay board + GPIO): locks/unlocks the battery unit mechanically

- 2× limit switches (top): detect whether the battery is fully seated/lifted

- ArUco markers on the sides (detected by BXU front camera) and on the bottom (detected by BXU docking cameras)

3. Node Architecture

The system is divided into two groups of nodes: those that run on the BXU Pi and those that run on the BRU Pi.

3.1 BXU Nodes

bxu_front_vision_node

Role: Perception for the front approach phase.
Inputs: Front USB camera feed.
Processing: Detects ArUco markers on the BRU sides, estimates lateral offset, yaw error, and distance to BRU.
Outputs (published topics):

- /bxu/front_vision/alignment — processed alignment errors (lateral, yaw, distance) as a custom message BxuAlignmentOutput

- /bxu/front_vision/raw_markers — raw marker detections (IDs, corners, poses) for logging/debugging

Design note: This node's interface is intentionally designed to be perception-method-agnostic. The alignment topic publishes a fixed message type regardless of whether detection comes from ArUco or, in the future, an ML model. A future bxu_front_ml_vision_node would publish to the same topic, making it a drop-in replacement.

bxu_docking_vision_node

Role: Perception for the precise docking phase (BXU sliding under BRU).
Inputs: Camera #0 and Camera #1 (upward-facing), calibration files, alignment zero offset.
Processing: Detects ArUco markers on the BRU underside, applies calibration and offset, computes lateral and yaw alignment error for centering BXU exactly under BRU.
Outputs (published topics):

- /bxu/docking_vision/alignment — processed docking alignment errors as BxuAlignmentOutput

- /bxu/docking_vision/raw_markers — raw marker detections for logging/debugging

Design note: Same perception-agnostic interface as the front vision node. Each vision node is independently startable and testable. Neither vision node sends any motion commands — it only publishes observations.

bxu_front_approach_controller_node

Role: Closed-loop motion controller for the front approach phase.
Inputs: Subscribes to /bxu/front_vision/alignment.
Processing: Computes velocity commands (vx, vy, omega) to drive the alignment errors to zero. Implements the PID/proportional control logic currently embedded in main.py for this phase. Publishes zero velocity when within "done" thresholds.
Outputs (published topics):

- /bxu/cmd_vel — geometry_msgs/Twist velocity command

Design note: This node only runs its control loop when activated by the supervisor. When inactive, it publishes zero velocity. The supervisor decides when to activate/deactivate controllers based on the FSM state.

bxu_docking_controller_node

Role: Closed-loop motion controller for the precision docking phase.
Inputs: Subscribes to /bxu/docking_vision/alignment.
Processing: Computes velocity commands to precisely center BXU under BRU. Different gain tuning and thresholds than the front approach controller, since the docking phase requires higher precision at lower speeds.
Outputs (published topics):

- /bxu/cmd_vel — geometry_msgs/Twist velocity command (same topic as above)

Design note: Both controllers publish to the same /bxu/cmd_vel topic. The supervisor ensures only one controller is active at any time (mutual exclusion through FSM state). The motion node does not know or care which controller is publishing — it just executes whatever comes through /cmd_vel.

bxu_motion_node

Role: Hardware interface for BXU locomotion (mecanum wheels).
Inputs: Subscribes to /bxu/cmd_vel.
Processing: Converts geometry_msgs/Twist (vx, vy, omega) to the V vx vy w\n serial command and writes it to the motion Pico over USB serial. Also handles stop and relax commands.
Outputs: None (serial write to Pico). Optionally publishes /bxu/motion/status for diagnostics.

Design note: This is the only node that has direct access to the motion Pico serial port. All motion in the system ultimately goes through this node. The separation between controller nodes and this node means controllers can be tested in simulation or with mock motion without touching hardware.

bxu_lift_node

Role: Hardware interface for BXU battery plate lifting/lowering mechanism.
Inputs: Listens on /bxu/lift/command topic or a ROS 2 service /bxu/lift (see Section 5 discussion).
Processing: Translates LIFT / LOWER commands to the lift Pico via USB serial (same serial pattern as motion Pico). Publishes current lift state.
Outputs (published topics):

- /bxu/lift/state — current state: IDLE, LIFTING, LIFTED, LOWERING, LOWERED

Design note: The lift mechanism is independent of locomotion and has its own Pico, so it gets its own dedicated node. This makes it testable in isolation.

bxu_supervisor_node

Role: Top-level FSM orchestrator for the BXU. The brain of the system.
Inputs: Subscribes to:

- /bxu/front_vision/alignment

- /bxu/docking_vision/alignment

- /bxu/lift/state

- /bru/lock/status (from BRU Pi)

- /bru/battery_seated (from BRU Pi)

- /bxu/safety/safe_to_move (future, from safety node)

Processing: Runs the FSM, transitions between states based on perception and hardware feedback, activates/deactivates controllers, calls BRU services, commands the lift node.
Outputs:

- Activates bxu_front_approach_controller_node or bxu_docking_controller_node (via parameter or activation topic)

- Calls /bru/lock_battery or /bru/unlock_battery services

- Publishes to /bxu/lift/command

- Publishes /bxu/supervisor/state (current FSM state as string, for monitoring/logging)

Design note: The supervisor contains FSM logic only — it never directly sends velocity commands or calls the Pico. It delegates all hardware interaction to the dedicated interface nodes.

3.2 BRU Nodes

bru_lock_node

Role: Hardware interface for the battery lock mechanism on the BRU.
Inputs: ROS 2 services /bru/lock_battery and /bru/unlock_battery.
Processing: On service call, actuates the lock motor via GPIO/relay. Uses a time-limited pulse to avoid stalling. Reads limit switch states.
Outputs (published topics):

- /bru/lock/status — current lock state: LOCKED, UNLOCKED, UNKNOWN

- /bru/battery_seated — std_msgs/Bool: True only when both top limit switches are activated simultaneously (battery is fully up and seated)

Design note: This node entirely replaces the Flask server used in the current setup. It exposes the same lock/unlock logic as proper ROS 2 services, so the BXU supervisor can call them directly over the network via DDS.

3.3 Future/Placeholder Nodes (not implemented yet, but designed into the architecture)

These nodes are anticipated and the topic names are reserved now so the architecture does not need restructuring later:

- bxu_safety_node: Subscribes to future safety sensors (lidar, bumpers, proximity). Publishes /bxu/safety/safe_to_move (std_msgs/Bool). The supervisor subscribes to this; if it's False, no motion commands are issued regardless of FSM state.

- bxu_front_ml_vision_node: Drop-in replacement for bxu_front_vision_node. Publishes to the same /bxu/front_vision/alignment topic using ML-based detection. No other node needs to change.

- bru_safety_node: Publishes sensor data from future BRU-side sensors (pressure, IR, camera) to /bru/... topics.

4. Full State Machine (FSM)

The FSM lives entirely in bxu_supervisor_node. The table below defines the complete swap sequence for battery insertion (delivering a battery to the BRU). Retrieval is the same sequence with steps 6–8 reversed as described below.

4.1 Battery Insertion Sequence

| # | State | Active Perception | Active Controller | Done Condition | Next State |
| --- | --- | --- | --- | --- | --- |
| 1 | SEARCHING | Front vision | None (manual/slow drive) | Markers detected | COARSE_APPROACH |
| 2 | COARSE_APPROACH | Front vision | Front approach controller | Distance ≤ coarse threshold, rough alignment | FINE_ALIGN |
| 3 | FINE_ALIGN | Front vision | Front approach controller | Yaw err ≤ fine_yaw_thresh AND lateral err ≤ fine_lat_thresh | SLIDE_UNDER |
| 4 | SLIDE_UNDER | (transition) | Front approach controller (blind forward) | Elapsed time / encoder / docking cams detected | DOCKING_ALIGN |
| 5 | DOCKING_ALIGN | Docking vision | Docking controller | Lateral + yaw within docking thresholds | LIFT_BATTERY |
| 6 | LIFT_BATTERY | None | None (lift command) | /bxu/lift/state == LIFTED | LOCK_BATTERY |
| 7 | LOCK_BATTERY | None | None (BRU service call) | /bru/lock/status == LOCKED | LOWER_PLATE |
| 8 | LOWER_PLATE | None | None (lift command) | /bxu/lift/state == LOWERED | DRIVE_AWAY |
| 9 | DRIVE_AWAY | None | Simple reverse command | Elapsed time / distance | IDLE |
| 10 | IDLE | None | None | — | — |

4.2 Battery Retrieval Sequence

States 1–5 are identical. From state 6:

| # | State | Action | Done Condition | Next State |
| --- | --- | --- | --- | --- |
| 6 | LIFT_EMPTY_PLATE | Lift plate (no battery on it) | /bxu/lift/state == LIFTED | UNLOCK_BATTERY |
| 7 | UNLOCK_BATTERY | Call /bru/unlock_battery service | /bru/lock/status == UNLOCKED AND /bru/battery_seated == False | LOWER_WITH_BATTERY |
| 8 | LOWER_WITH_BATTERY | Lower plate (now carrying battery) | /bxu/lift/state == LOWERED | DRIVE_AWAY |
| 9 | DRIVE_AWAY | Reverse | Elapsed time | IDLE |

4.3 Safety Rule

In any state where motion is active, if /bxu/safety/safe_to_move is False (when that node exists), the supervisor immediately transitions to EMERGENCY_STOP and publishes zero velocity. Recovery from EMERGENCY_STOP is manual.

5. Topics and Services Reference

Published Topics

| Topic | Message Type | Published by | Subscribed by |
| --- | --- | --- | --- |
| /bxu/front_vision/alignment | swas_msgs/BxuAlignmentOutput | bxu_front_vision_node | bxu_supervisor_node, bxu_front_approach_controller_node |
| /bxu/front_vision/raw_markers | swas_msgs/MarkerDetectionArray | bxu_front_vision_node | (logging/debug only) |
| /bxu/docking_vision/alignment | swas_msgs/BxuAlignmentOutput | bxu_docking_vision_node | bxu_supervisor_node, bxu_docking_controller_node |
| /bxu/docking_vision/raw_markers | swas_msgs/MarkerDetectionArray | bxu_docking_vision_node | (logging/debug only) |
| /bxu/cmd_vel | geometry_msgs/Twist | bxu_front_approach_controller_node OR bxu_docking_controller_node | bxu_motion_node |
| /bxu/lift/command | std_msgs/String | bxu_supervisor_node | bxu_lift_node |
| /bxu/lift/state | std_msgs/String | bxu_lift_node | bxu_supervisor_node |
| /bxu/supervisor/state | std_msgs/String | bxu_supervisor_node | (monitoring/logging) |
| /bxu/motion/status | std_msgs/String | bxu_motion_node | (diagnostics) |
| /bru/lock/status | std_msgs/String | bru_lock_node | bxu_supervisor_node |
| /bru/battery_seated | std_msgs/Bool | bru_lock_node | bxu_supervisor_node |
| /bxu/safety/safe_to_move | std_msgs/Bool | bxu_safety_node (future) | bxu_supervisor_node |

Services

| Service | Type | Server | Called by |
| --- | --- | --- | --- |
| /bru/lock_battery | std_srvs/Trigger | bru_lock_node | bxu_supervisor_node |
| /bru/unlock_battery | std_srvs/Trigger | bru_lock_node | bxu_supervisor_node |

Note on service transport across Pis: ROS 2 uses DDS (Data Distribution Service) as its middleware. When both Pis are on the same network and using the same ROS_DOMAIN_ID, topics and services are automatically discovered and routed across the network without any additional configuration. No additional bridge or proxy (like Flask) is needed.

6. Custom Message Types

Custom messages are defined in a sibling package swas_msgs (part of the same repository and workspace).

swas_msgs/BxuAlignmentOutput.msg

# Alignment error output from any BXU vision node
std_msgs/Header header

float32 lateral_error_m       # Lateral offset from target, in metres (positive = right)
float32 yaw_error_rad         # Yaw misalignment, in radians (positive = clockwise)
float32 distance_m            # Estimated distance to target, in metres (-1 if unknown)

bool markers_detected         # True if at least one valid marker is detected
int32 num_markers_detected    # Number of markers contributing to this estimate
string visibility_class       # e.g. "NONE", "PARTIAL", "FULL" — for FSM decisions
float32 confidence            # 0.0 to 1.0 — quality of current estimate

swas_msgs/MarkerDetectionArray.msg

# Raw marker detection list, for logging and debugging
std_msgs/Header header
swas_msgs/MarkerDetection[] detections

swas_msgs/MarkerDetection.msg

int32 marker_id
float32[4] corners_u          # pixel u-coordinates of corners
float32[4] corners_v          # pixel v-coordinates of corners
geometry_msgs/Pose pose       # 3D pose in camera frame

7. Package and File Structure

The repository contains two ROS 2 packages: swas_msgs (interface definitions) and swas_system (all executable nodes).

SWAS-DEMO-NL-TEAM/                          ← git repository root
├── README.md
├── .gitignore
│
└── ros2_ws/
    └── src/
        │
        ├── swas_msgs/                  ← Custom message/service definitions
        │   ├── package.xml
        │   ├── CMakeLists.txt
        │   └── msg/
        │       ├── BxuAlignmentOutput.msg
        │       ├── MarkerDetection.msg
        │       └── MarkerDetectionArray.msg
        │
        └── swas_system/                ← All executable nodes
            ├── package.xml
            ├── setup.py
            ├── setup.cfg
            │
            ├── config/
            │   ├── bxu_params.yaml         ← All tunable parameters (thresholds, gains, serial ports)
            │   └── bru_params.yaml
            │
            ├── launch/
            │   ├── bxu_demo.launch.py      ← Launch all BXU nodes for real robot demo
            │   ├── bru_demo.launch.py      ← Launch all BRU nodes
            │   └── sim.launch.py           ← Launch simulation (future)
            │
            ├── calib/
            │   ├── alignment_zero.npz
            │   ├── camera_0_1280x720_usb_charuco_calibration.npz
            │   ├── camera_0_640x480_calibration.npz
            │   └── camera_1_640x480_calibration.npz
            │
            └── swas_system/
                ├── __init__.py
                │
                ├── bxu/
                │   ├── __init__.py
                │   ├── front_vision_node.py        ← bxu_front_vision_node
                │   ├── docking_vision_node.py      ← bxu_docking_vision_node
                │   ├── front_approach_controller_node.py
                │   ├── docking_controller_node.py
                │   ├── motion_node.py              ← bxu_motion_node (serial to Pico)
                │   ├── lift_node.py                ← bxu_lift_node (serial to lift Pico)
                │   └── supervisor_node.py          ← bxu_supervisor_node (FSM)
                │
                ├── bru/
                │   ├── __init__.py
                │   └── lock_node.py                ← bru_lock_node (GPIO relay + limit switches)
                │
                └── common/
                    ├── __init__.py
                    └── global_config.py            ← Shared constants (replaces old global_config.py)

8. Node-to-Existing-Script Mapping

This table shows exactly how each existing Python script in ~/SWAS-DEMO-NL-Team/bxu_pi maps to the new ROS 2 node(s), making clear what logic moves where.

| Existing Script | Logic Moves To | Notes |
| --- | --- | --- |
| front_cam_vision.py | bxu/front_vision_node.py | Wraps detection logic; publishes alignment + raw topics instead of returning values |
| docking_vision.py | bxu/docking_vision_node.py | Same pattern; handles dual-camera setup and offset correction |
| mecanum_controller.py | bxu/motion_node.py | Serial write logic becomes the node; receives /bxu/cmd_vel instead of being called directly |
| main.py (FSM part) | bxu/supervisor_node.py | State machine logic only; no direct hardware calls |
| main.py (front approach PID) | bxu/front_approach_controller_node.py | Control loop for front phase |
| main.py (docking PID) | bxu/docking_controller_node.py | Control loop for docking phase |
| main.py (lift commands) | bxu/lift_node.py + supervisor | Lift hardware interface extracted to dedicated node |
| global_config.py | common/global_config.py + config/bxu_params.yaml | Constants move to ROS 2 parameter file; global_config.py kept as Python fallback for shared utilities |
| calib/*.npz | calib/ (same) | Copied into package; path resolved relative to package install |
| calibrate_camera.py | Not a node — keep as standalone script | Calibration helper, does not need to be a ROS node |
| calibrate_usb_camera.py | Not a node — keep as standalone script | Same |

9. Parameter Management

All tunable values that currently live in global_config.py (thresholds, gains, camera IDs, serial port paths) should be moved to config/bxu_params.yaml. ROS 2 allows loading YAML parameters at launch time, which means tuning does not require editing Python files.

Example config/bxu_params.yaml structure:

bxu_front_vision_node:
  ros__parameters:
    camera_device: "/dev/video0"
    calib_file: "calib/camera_0_1280x720_usb_charuco_calibration.npz"
    marker_ids: [0, 1, 2, 3]

bxu_front_approach_controller_node:
  ros__parameters:
    kp_lateral: 0.8
    kp_yaw: 0.5
    kp_distance: 0.4
    fine_lateral_thresh_m: 0.01
    fine_yaw_thresh_rad: 0.02

bxu_supervisor_node:
  ros__parameters:
    mode: "insertion"   # "insertion" or "retrieval"

10. Multi-Pi ROS 2 Networking

Both Pis communicate over the same LAN (WiFi or Ethernet). ROS 2 DDS auto-discovers all nodes, topics, and services on the same network without any configuration, as long as:

- Both Pis have ROS 2 installed and the same workspace sourced.

- Both use the same ROS_DOMAIN_ID (set in ~/.bashrc on both Pis, e.g., export ROS_DOMAIN_ID=42).

- No firewall blocks UDP multicast on the local network.

With this in place, the BXU supervisor calls /bru/lock_battery exactly the same way regardless of whether the BRU node is on the same machine or a different machine. DDS handles the transport transparently.

11. Launch Strategy

Three launch files are planned:

bxu_demo.launch.py — Run on BXU Pi for real robot demo:

- Starts: bxu_front_vision_node, bxu_docking_vision_node, bxu_front_approach_controller_node, bxu_docking_controller_node, bxu_motion_node, bxu_lift_node, bxu_supervisor_node

- Loads: config/bxu_params.yaml

bru_demo.launch.py — Run on BRU Pi:

- Starts: bru_lock_node

- Loads: config/bru_params.yaml

sim.launch.py (future) — Run on development machine:

- Starts: Gazebo, robot model, mock motion/lift nodes, all BXU perception and supervisor nodes

- Loads: sim-specific parameter overrides

12. Migration Order (Recommended Implementation Sequence)

Rather than building everything at once, the following order minimises risk and keeps a working system at every step.

Step 1: Create workspace and package skeleton
Set up ros2_ws/, create swas_msgs and swas_system packages, define the custom message types, and verify colcon build succeeds.

Step 2: Implement and test bxu_motion_node
Port mecanum_controller.py to a ROS 2 node that subscribes to /bxu/cmd_vel and writes serial commands to the Pico. Test by publishing velocity commands manually with ros2 topic pub.

Step 3: Implement and test vision nodes
Port front_cam_vision.py → front_vision_node.py and docking_vision.py → docking_vision_node.py. Verify they publish correct alignment data on their topics using ros2 topic echo.

Step 4: Implement controller nodes
Extract the two PID/proportional control loops from main.py into front_approach_controller_node.py and docking_controller_node.py. Test each in isolation: start the vision node, start the controller, and check that it publishes sensible /bxu/cmd_vel values.

Step 5: Implement bxu_lift_node
Port lift Pico communication. Test lift and lower commands in isolation.

Step 6: Implement bxu_supervisor_node
Port the FSM from main.py. Start with states 1–5 (approach and docking only) and verify the full approach sequence runs correctly from a single launch. Add states 6–9 once the approach chain is stable.

Step 7: Implement bru_lock_node
Port BRU lock/unlock logic. Test the service call from BXU Pi (ros2 service call /bru/lock_battery) while both Pis are on the same network.

Step 8: Full integration test
Run bxu_demo.launch.py on BXU Pi and bru_demo.launch.py on BRU Pi simultaneously. Execute a complete swap sequence end-to-end.

Step 9: Add Gazebo simulation (separate effort)
Build BXU URDF model, add Gazebo differential/mecanum drive plugin, create sim.launch.py. Target: run supervisor + controller nodes unchanged in simulation.

13. Open Questions / Design Decisions for Later

- Controller activation mechanism: The current spec assumes the supervisor controls which controller is active via a topic or parameter. A cleaner ROS 2 pattern would be using Lifecycle Nodes (each controller is a managed node that the supervisor activates/deactivates). This is an optional upgrade for a later iteration.

- /bxu/cmd_vel arbitration: Two controller nodes publish to the same topic. Since only one is active at a time (enforced by the supervisor), there is no conflict in practice. If stricter arbitration is needed later, a mux node (e.g., topic_tools/mux) can be inserted.

- Lift Pico serial protocol: The current spec assumes the lift Pico uses a similar serial command protocol to the motion Pico. This needs to be verified and the protocol documented before implementing bxu_lift_node.

- Simulation fidelity: First Gazebo iteration uses differential drive approximation for mecanum. Proper mecanum physics requires a custom Gazebo plugin or the use of a simulator with native mecanum support.